IMAGE PROCESSING & EDGE DETECTION USING VIVADO TOOL:

Step1: Line Buffer Design :

- One line buffer will be going to store one line of the image.

- It is some kind of RAM where data will be initially filled from the original image and data will be read out & processed later.

- Each pixel is only one byte.

Writing the Verilog code for line buffer and then we have to perform simulation:

Code_Link LineBuffer.v code

Simulation of line Buffer :

Note: Xilinx provide different kind of Ips ,but as per our requirement we have to write our own IP because we can customise them as per our need.

Step2: Design of module which will do multiplication and summation operation.

[MAc module Multiplication & Accumulation]

- This module will get the pixels & it will also get the kernels & it will basically multiply pixels with corresponding kernel value & add them together.

- Writing the verilog code for the module.

Code_Link conv.v code

Step3: Design of Control Logic.

- Design a module which instantiate all the line buffers and these multiplexers ,

Which control storing the data into line buffers as well as which control sending data from these module to the MAc module

- Here we will write our imageController verilog code.

Code_Link imageControl.v code

Step4:   IP Packaging:

- We will combine all the modules we have developed till now and we will complete hardware design for our IP. So that we can package our own IP.

Code_Link ImageProcessTop.v  code

- Then click IPCatalog   select FIFO Generator  we are going to use AXI stream FIFO click OK click Generate.

- Instantiate this into my ImageProcessTop.v file      Code_Link ImageProcessTop.v code

- Now we will convert it into IP AXI format . So that we can use this IP in our block design.

- Click on       Tools Create Package IP Package your current project NextFinish

Step 5: Simulation.

- Till now we have packaged our IP but not yet verified.So we have to do simulation and we cannot verify it by waveform here.

- We have to realy send an image(.bmp format) & check whether the output is coming properly or not.

- So we have to write our own test bench [which will read the image & send it to our IP and it will receive the data coming from  the IP and I will be able to see the actual output.]

- Code_Link  tb.v file

- After that go to     settings simulation OKrun simulation

Input image   .bmp file    output  blurred image .bmp file

Step 6: System Integration

- We are going to build a system which is going to use this IP core.

- Create new project   create  block design .Select the following IP:

- Zynq Processing System | our IP(ImageProcessTop_v1) |AXI DMA

- Then run connect automation  | after that select some more IPs like AXI4 - Data width Converter| AXI Interconnect | Concat IP  then again run connect automation

- Verify design

- Generate Wrapper [Sorce select ImageProcessingSystemcreate HDL wrapper]

- Generate BitStream

Log window: showing successful synthesis completion

Timing Checks: Setup time & Hold Time

Power Analysis from the implemented netlist:

Step7: Software Development & Hardware Evaluation

- We have completed the hardware design part of our system which uses our image processing IP. Next we have to go to software part of the system in order to verify it on hardware.

- An image is simply a big array of numbers , so instead of sending image to external world we can create a static array of that big numbers & make it a part of our software source code.

- Verilog can do filehandling also. So I will modify my testbench(tb.v) file to get the header file (imageData.h).

Link_of_imageData.h_file

- Export hardware & launch vitis IDE tool.

- Create a new application project & import file imageData.h file .Next go to source & create file imageTest.c file.

- Link_of_imageTest.c_file

Now we will test it on hardware.

- Go to vivado & program the device

Now actually the data is coming & the entire processing is over . It is sending back the processed data .Now we will get our image filtered output similar to simulation.

Edge Detection Image :

Image must be stored in .bmp format

The edge detected image using Solebel